Step-by-Step Instructions

(PRINT this set of instructions to follow every time you refill your cartridge)

Hewlett Packard #901 (CC656A) Color

Items Needed: gloves, cyan, yellow & magenta ink, hand-drill, #5 refill station, paper towels, review hints & tips and check list  Do not plug fill holes.

Bottle Seal:   Unscrew lid on  ink bottle.  Puncture  silver seal with  needle  top making a small hole in the center of seal. Tightly screw on dispensing tip. Rinse dispenser tip before using dispenser tip with another color. Rinse tip with water until the water runs clear. Pull plastic tip cover off of tip of syringe to pour ink.

1.) Place cartridge in Station #5 with copper circuits facing to the left and the plastic feet inside the bottom of the station to your right. Refill station lid has three holes. Place the lid on cartridge with single hole on the left and double holes on the right. Make sure lid snaps down over top of cartridge.

2.) One by one, insert drill into each of the three holes and down into the inkjet cartridge. You will have three fill holes for this cartridge. Rinse and dry drill after inserting into each color chamber.

3.) Remove the lid from cartridge. Leave cartridge in refill station.

4.) Insert the correct color bottle tip deep inside of each of the fill-holes. See picture and fill colors as shown.  S-l-o-w-l-y dispense up to 5 ml of ink into each of the three fill- holes. If ink begins to “bubble” out of the fill-hole, remove bottle tip. As you inject the ink withdraw and re-insert the syringe at different angles in order to expose the whole sponge to the ink. Bubbling occurs when a cartridge is either over-filled or if ink is being inserted too quickly. Wait several minutes and then try to slowly insert more ink. If ink begins to bubble out the fill-hole or leak from the printhead, you have plenty of ink in the

cartridge. A little ink might come out of the top when full, so cover up the other holes while filling one.

5.) After refilling, remove cartridge from refill station. Without the cartridge “printhead” touching the paper towels, place refilled cartridge upright upon folded paper towels and tilt cartridge at a slight angle. If the printhead on the cartridge is touching the paper towel, it could create a wicking effect which would pull the ink from your refilled cartridge. It is necessary for your cartridge to sit for at least 20 minutes after refilling because there maybe some minor leakage from the cartridge. Never put a leaking cartridge back into your printer.

6.) Do not plug fill-holes.

7.) Check the ink flow by gently pressing a dry paper towel against the printhead. You should see 3 small lines of each color. If not, gently press a warm, wet paper towel against the printhead until each color is visible.

8.) When all 3 colors are visible on paper towel, place cartridge back into the printer and run 1-2 print tests. Or print 2 pages at this test site:

http://www.freeink4life.com/support/black_print_test.html  Black ink test http://www.freeink4life.com/support/color_print_test.html  Color ink Test

For best results, leave cartridge in printer for 8 hours after refilling so the ink can saturate the entire sponge.

AccuBrite, Inc. is not affiliated with the listed printer manufacturers or their subsidiaries. The use by AccuBrite, Inc. does not imply endorsement by or association with the brand name owners. The listed manufacturers, printers and cartridges are used only for the purpose of showing the compatibility of this AccuBrite, Inc. product. All other trademarks, brand names and logos referenced in or on this product are the property of their respective owners. This material is protected by the copyright laws of the United States and other countries. It is illegal to reproduce these instructions. It may not be reproduced, distributed, or altered in any fashion by any entity (either internal or external to AccuBrite), except  in  accordance  with  applicable  agreements  or  contracts  of  licensing, without the express written consent of AccuBrite, Inc.

Copyright © AccuBrite, Inc. 2003-2008. All rights reserved